# Belden Autonomous Operations
Agentic Workflow Design

Workflow-driven multi-agent orchestration for BRS/SFP anomaly resolution

# 1. Recommended Architecture

Use a workflow-driven multi-agent system. The workflow/orchestration layer owns control, state, retries, branching, approvals, and execution. Specialized agents provide reasoning at specific stages.

Event
  ↓
Workflow / Orchestrator
  ↓
Correlation Agent
  ↓
RCA Agent
  ↓
Recommendation Agent
  ↓
Policy Engine
  ↓
Executor
  ↓
Verification

Key principle: The workflow controls the agents; agents do not directly control the entire incident lifecycle.

# 2. Example Event

Assume the orchestration layer receives the following anomaly:

BRS-101
Component: SFP-2
Anomaly: SFP degradation
RX optical power: decreasing
CRC errors: increasing
Severity: High

# 3. What Each Step Does

| Step | Primary Question | What It Does | Typical Output |
| --- | --- | --- | --- |
| 1. Workflow / Orchestrator | What should happen next? | Creates/loads the incident, controls sequence, invokes agents, stores state, handles retries/timeouts and branching. | Incident INC-1001; next step = Correlation |
| 2. Correlation Agent | What else is related? | Enriches the anomaly with recent telemetry, topology, connected ports/devices, alarms, configuration changes and historical incidents. | Both ends of the same link show optical degradation |
| 3. RCA Agent | Why is this happening? | Evaluates the correlated evidence and ranks likely root causes with evidence and confidence. | Fiber path degradation, confidence 82% |
| 4. Recommendation Agent | What should we do? | Uses the RCA result plus SOPs, KBAs, manuals and prior resolutions to generate an ordered remediation plan. | Run optical diagnostic → inspect fiber → replace SFP if required |
| 5. Policy Engine | Are we allowed to do it? | Applies deterministic rules for risk, approvals, maintenance windows, SLA and permitted actions. | Diagnostic = auto; SFP replacement = approval required |
| 6. Executor | Carry out the approved action. | Calls device-management, ServiceNow, RMA, inventory, logistics or notification APIs. | Diagnostic run / ticket created / RMA initiated |
| 7. Verification | Did remediation actually work? | Re-checks telemetry and health after action completion; closes, retries, re-runs diagnosis or escalates. | RX power normal; CRC errors stable; incident resolved |

# 4. Detailed Example

## 4.1 Correlation Agent

Input: BRS-101 / SFP-2 degradation event.

Fetch / inspect:
- Last 15–30 minutes of SFP and port telemetry
- Connected neighbor port/device
- Network topology
- Other active alarms
- Recent configuration changes
- Similar historical incidents

Result:
BRS-101 SFP-2 RX power ↓
+
BRS-102 SFP-1 RX power ↓
+
Both ports are on the same physical link
+
No recent configuration change

Conclusion: the anomaly is likely related to the shared optical path, not only to one isolated port.

## 4.2 RCA Agent

The RCA Agent receives the original event plus the correlated context.

Possible causes:
1. Fiber path degradation      82%
2. SFP transceiver failure     13%
3. Switch port failure          5%

Selected root cause:
FIBER_PATH_DEGRADATION
Confidence: 0.82

## 4.3 Recommendation Agent

The Recommendation Agent retrieves the relevant Belden operational knowledge and produces a remediation plan.

1. Run optical diagnostic
2. Inspect / clean fiber connector
3. Re-check RX/TX optical levels
4. Reseat SFP if needed
5. Replace SFP if diagnostics still fail

## 4.4 Policy + Execution

RUN_OPTICAL_DIAGNOSTIC   → AUTO ALLOWED
READ_TELEMETRY            → AUTO ALLOWED
RESTART_PRODUCTION_PORT    → APPROVAL REQUIRED
REPLACE_SFP                → APPROVAL REQUIRED
FIRMWARE_UPGRADE           → MAINTENANCE WINDOW + APPROVAL

The orchestrator executes only the actions permitted by policy. If approval is required, the workflow pauses, persists state, waits for approval, then resumes.

## 4.5 Verification

After remediation:
- Is RX optical power back within acceptable range?
- Have CRC errors stopped increasing?
- Is the link stable?
- Has the alarm cleared?

YES → Resolve incident
NO  → Try next approved action / re-run RCA / escalate to human

# 5. Workflow or Multi-Agent System?

The recommended design is both, but with strict separation of responsibility:

| Layer | Owns | Examples |
| --- | --- | --- |
| Workflow / Orchestrator | Control and lifecycle | State, retries, branching, approval, timeout, pause/resume |
| Agents | Reasoning | Correlation, root-cause analysis, remediation recommendation |
| Policy Engine | Governance | Allowed actions, risk rules, SLA, approvals |
| Executors | Deterministic action | Device API, ServiceNow, RMA, inventory, logistics |

Recommended interaction pattern:

Workflow → Correlation Agent → result
Workflow → RCA Agent → result
Workflow → Recommendation Agent → result
Workflow → Policy Engine → decision
Workflow → Executor → result
Workflow → Verification → close / retry / escalate

Avoid: Correlation Agent directly calling RCA Agent, which directly calls Recommendation Agent, with no workflow control between steps.

# 6. Final Design Principle

For the Belden autonomous-operations use case, the safest and most auditable architecture is a deterministic workflow/state machine with specialized agents embedded as bounded reasoning steps.