BRS50 Switch — Anomaly & Data Source Mapping

Target model provided: BRS50-00122Q2Q-SFCZ99HHSEA

# Purpose and scope

This document maps candidate switch/network anomalies to the data needed to detect them, the official source link, and the relevant manual section/feature. It is intended to support Belden Horizon network analysis. These are operational/diagnostic anomalies, not proof of product end-of-life (EOL) or end-of-support (EOS). Lifecycle dates must be sourced separately from official product notices and asset records.

# Official source links

S1 — Official Hirschmann documentation portal (BRS manuals; select BRS and matching HiOS variant/version): https://www.doc.hirschmann.com/

S2 — Official Belden product datasheet/configuration page (BRS50-8TX/4SFP; similar product-code family, exact suffix must be confirmed): https://catalog.belden.com/index.cfm?configProduct=BRS50-00122Q2Q-STCY99HHSESXX.X.XX&event=generateTDS

S3 — Official HiOS feature overview: https://www.doc.hirschmann.com/hios.html

S4 — Official HiOS CLI Reference Manual, Release 10.4 overview (general HiOS CLI reference; confirm compatibility with device firmware): https://www.doc.hirschmann.com/pdf/RM_CLI_HiOS-10400_Overview_en.pdf

S5 — Official Belden/Hirschmann support center (product notices/support enquiries): https://hirschmann-support.belden.com/

# Anomaly mapping

| Anomaly candidate | Data required | Source link | Relevant section / why it supports detection |
| --- | --- | --- | --- |
| Repeated link up/down (link flap) | Port ID; link state changes; event timestamps; count of changes within an observation window. | S1, S2, S3 | HiOS manual: Diagnostics > Ports > Port Monitor > Link flap. Feature overview lists Link Flap Detection. Repeated link changes are evidence of an unstable link; the manual feature identifies the condition, while thresholds/configuration must be checked in the matching manual. |
| CRC / fragment error burst | Per-port CRC/FCS errors and fragments; counter deltas over time; port ID; timestamps. | S1, S2, S4 | HiOS manual: Diagnostics > Ports > Port Monitor > CRC/Fragments; CLI reference includes auto-disable reason `crc-error`. Rising errors can indicate a physical/link-quality issue, but root cause needs further diagnosis. |
| Port overload / unusual traffic load | Per-port traffic/load counters; configured overload-detection parameters; event/alarm logs; timestamps. | S1, S2, S3, S4 | HiOS manual: Diagnostics > Ports > Port Monitor > Overload detection. CLI reference: auto-disable reason `overload-detection`. Supports detecting configured data-load overload, not automatically classifying malicious traffic. |
| Duplex mismatch | Port speed; duplex mode at both ends where available; mismatch events; port ID and timestamps. | S1, S2, S3, S4 | HiOS manual: Diagnostics > Ports > Port Monitor > Duplex mismatch detection. Feature overview lists Duplex Mismatch Detection. A mismatch is a documented port diagnostic condition. |
| Unexpected link speed/duplex change | Negotiated link speed; duplex mode; configured/allowed combination; change events. | S1, S2, S3, S4 | HiOS manual: Diagnostics > Ports > Port Monitor > Link speed/Duplex mode detection. Detects an unpermitted combination according to configured parameters. |
| SFP/module warning or optical diagnostics | SFP presence; vendor/type; compatibility/support status; supported optical/temperature values if exposed by this SFP and firmware; port ID; logs. | S1, S2, S3 | HiOS manual: Diagnostics > Ports > SFP / SFP Management. Datasheet lists SFP Management. Exact readable fields depend on SFP type and firmware; verify against the BRS HiOS manual and actual switch output before assuming TX/RX power or temperature is exposed. |
| Copper cable fault (where supported) | Cable diagnosis result per port; open/short status; fault distance if returned. | S1, S2, S3 | HiOS manual: Diagnostics > Ports > TP cable diagnosis / Copper Cable Test. Datasheet lists Copper Cable Test. Availability depends on the physical port/media and model. |
| System/device health or self-test warning | Device status; hardware state; self-test result; alarm/event records; timestamps. | S1, S2 | HiOS manual: Diagnostics > System > System Information / Hardware State / Selftest (exact headings vary by HiOS version). Datasheet lists System Information and Self-Tests on Cold Start. Use returned status/events as evidence; do not infer an unreported internal fault. |
| Operating temperature outside specification (environmental risk) | Measured ambient temperature near switch; configured alarm/threshold if available; timestamp; product's rated operating range. | S1, S2 | Installation manual: Technical data / Ambient conditions. Datasheet lists operating temperature 0 to +60 °C for the referenced BRS50 configuration. Treat this as an environmental limit, not proof the switch reports an internal temperature sensor. |
| Lifecycle EOL/EOS approaching (separate lifecycle indicator) | Exact product code; serial number; official EOL/EOS notice/date; installed firmware; asset age and support contract. | S1, S5 + asset/vendor lifecycle records | Not established by the diagnostic sections above. Use official product lifecycle notices/support confirmation and asset/contract records. A switch manual alone generally does not provide the exact EOL/EOS date for a particular SKU. |

# Important validation notes

The Belden product page found is for a closely related BRS50-8TX/4SFP product-code family, not an exact confirmed match for the full suffix provided. Confirm the full SKU/configuration before applying port/media capabilities or thresholds.

Use the official documentation portal to select the BRS50 manual collection matching the device's actual HiOS variant (for example, HiOS-2A or HiOS-2S) and firmware release. The section labels above are navigation targets; exact page numbers can differ by release.

Manuals describe supported diagnostics; they do not guarantee that each field is present in exported telemetry, SNMP, syslog, or S3 data. Verify by checking the switch's live UI/CLI and actual logs.

For lifecycle intelligence, combine device inventory and firmware data with official EOL/EOS/product notices, warranty records, and service history. Do not label ordinary link/CRC/SFP events as EOL/EOS evidence.

Suggested initial rule examples (tune with site data): repeated link transitions in a time window; rapid increase in CRC errors; configured overload event; unpermitted speed/duplex combination; cable diagnosis failure; system alarm/self-test failure; temperature beyond the rated operating range. Avoid fixed thresholds unless the manual/configuration or operational baseline supports them.

Reference status: Links and feature names were checked against official Hirschmann/Belden documentation search results. Exact SKU-specific limits and page numbers should be confirmed against the installed switch's firmware/manual release.